CHAPTER 5

CONCLUSION AND FUTURE WORK

5.1  Summary of Contributions

This project has presented a comprehensive computational framework for the parametric analysis and optimization of bifacial photovoltaic systems. The work addresses the identified gaps in existing literature by providing a systematic, accessible, and location-specific tool for determining optimal installation configurations for bifacial PV modules. The key contributions of this work are summarized as follows.

First, a rigorous mathematical model for computing the front-side and rear-side irradiance on bifacial PV modules has been implemented. The model incorporates the Liu-Jordan isotropic sky model [1] for front-side irradiance estimation, the Erbs clearness-index correlation for diffuse-direct decomposition, and the Yusufoglu view factor approach [2] for accurate rear-side irradiance calculation with shadow correction. The formulation accounts for the combined effects of tilt angle, ground albedo, mounting height, solar geometry, and panel azimuth on both front-side and rear-side irradiance contributions, providing a physically consistent representation of bifacial PV module operation under realistic conditions.

Second, a MATLAB/Simulink model has been developed for simulating the electrical characteristics of bifacial PV modules. The Simulink model utilizes a single-diode equivalent circuit representation of the PV module and incorporates the NOCT-based thermal model for temperature derating. The model enables the computation of complete I-V and P-V characteristic curves under any combination of irradiance and temperature conditions, providing validated electrical output predictions for system design and performance assessment.

Third, a user-oriented frontend platform has been designed and implemented that integrates location-based solar irradiance data retrieval from the NASA POWER API with automated parametric sweep analyses. The platform features a modular architecture comprising a responsive web-based user interface, a Node.js/Express backend application layer, and dual analysis engines (JavaScript-based and MATLAB-based). The platform provides an accessible tool for PV system designers that requires no specialized software knowledge or access to commercial simulation tools, directly addressing the accessibility gap identified in the literature review.

Fourth, the framework supports dual optimization objectives, namely maximum total energy generation and maximum rear-side gain. This dual optimization capability enables system designers to evaluate the trade-offs between these complementary objectives and make informed configuration decisions based on their specific project requirements and priorities. Few existing tools in the literature provide this level of optimization flexibility for bifacial PV systems.

Fifth, the proposed approach has been validated through a detailed case study for Greater Noida, India (28.47°N, 77.50°E), with results demonstrating good quantitative agreement with published literature including the parametric analysis of Dincer and Ozer [3] and the simulation study of Yusufoglu et al. [2]. The validation confirms the reliability and accuracy of both the mathematical formulation and its computational implementation across diverse installation configurations.

5.2  Key Findings

The parametric analyses conducted in this study have yielded the following key findings that provide practical guidance for the design and optimization of bifacial PV installations:

(i) Optimal Configuration for Maximum Energy: For the case study location (Greater Noida, India) with a concrete ground surface (albedo = 0.3), the optimal configuration for maximum energy output was identified as a tilt angle of 30 degrees and mounting height of 450 cm, achieving a total daily energy of 3.168 kWh per module and a rear gain of 16.55%. The optimal tilt angle of 30 degrees closely corresponds to the site latitude (28.47°N), consistent with established PV design principles for annual energy maximization.

(ii) Optimal Configuration for Maximum Rear Gain: The optimal configuration for maximum rear gain was identified as a tilt angle of 10 degrees and height of 450 cm, achieving a rear gain of 17.90% but with a lower total energy of 2.994 kWh (approximately 5.5% lower than the maximum energy configuration). This finding demonstrates the existence of a clear and quantifiable trade-off between total energy maximization and rear gain maximization that must be considered during system design.

(iii) Dominant Influence of Albedo: Ground surface albedo was confirmed as the most influential individual parameter affecting rear-side irradiance contribution. The rear gain varied from below 5% for low-albedo surfaces (such as dry asphalt with albedo approximately 0.12) to over 24% for highly reflective surfaces (such as aluminum with albedo 0.85), representing a nearly five-fold variation. The relationship between albedo and rear gain was found to be approximately linear, consistent with the mathematical formulation in equation (10) and independently confirmed by the simulation results of Yusufoglu et al. [2]. This finding has direct practical implications for site preparation and ground surface selection in bifacial PV installations.

(iv) Logarithmic Height Dependence: Mounting height exhibited a positive but diminishing effect on rear-side performance, consistent with a near-logarithmic dependence reported by Yusufoglu et al. [2]. The practical implication is that mounting heights in the range of 200-300 cm provide a good balance between performance improvement and structural feasibility for most ground-mounted applications. Beyond this range, the marginal energy benefit becomes insufficient to justify the additional structural cost and complexity.

(v) Necessity of Combined Optimization: The interaction between tilt angle, height, and albedo necessitates combined parametric optimization rather than independent single-parameter studies. The combined sweep approach captured interaction effects that would be missed by analyzing parameters in isolation. Furthermore, the system performance was found to be relatively robust to small variations near the optimal configuration, with the top five configurations differing by less than 0.25% in total energy, providing practical flexibility for accommodating site-specific constraints.

(vi) Consistency with Published Literature: The comparative analysis with published studies confirmed that the proposed framework produces results that are quantitatively consistent with established research. The rear gain values, albedo sensitivity trends, height dependency patterns, and tilt angle optimization results all showed good agreement with the reference studies of Dincer and Ozer [3], Yusufoglu et al. [2], Ganesan et al. [4], and Pelaez et al. [5], validating the accuracy and reliability of the implemented mathematical models.

5.3  Limitations

The present work has certain limitations that should be acknowledged and considered when interpreting the results and applying the framework to real-world system design:

(i) Uniform Albedo Assumption: The mathematical model assumes uniform ground albedo beneath and surrounding the module. In real-world installations, ground surface properties may vary spatially due to vegetation growth, shadows from nearby structures, mixed surface types, or degradation of artificial reflective surfaces over time. Non-uniform albedo distributions can lead to spatially varying rear-side irradiance that is not captured by the present model.

(ii) Single-Day Analysis: The case study analysis was performed using single-day irradiance data from the NASA POWER API. While this approach provides useful insights into the relative performance of different configurations and enables rapid optimization, extending the analysis to annual energy yield computations incorporating seasonal variations in solar geometry, temperature, and irradiance conditions would provide more comprehensive and robust optimization guidance for long-term system design decisions.

(iii) Single-Row Configuration: The current implementation models a single isolated bifacial PV module and does not account for inter-row shading effects that occur in multi-row array configurations. In large-scale solar installations, adjacent rows cast shadows on the ground between them, reducing the ground-reflected radiation available to the rear surface and potentially altering the optimal configuration parameters.

(iv) Simplified Thermal Model: Temperature effects on module performance are modeled using the simplified NOCT approach, which relates cell temperature to ambient temperature and incident irradiance through a linear approximation. More detailed thermal models that account for wind speed, convective cooling, mounting configuration effects on rear-surface ventilation, and radiative heat exchange may improve the accuracy of energy yield predictions, particularly in hot climatic conditions.

(v) Isotropic Sky Model: The Liu-Jordan isotropic sky model used for diffuse radiation estimation assumes uniform distribution of diffuse radiation across the sky hemisphere. This assumption may underestimate the circumsolar and horizon brightening components that contribute to tilted surface irradiance, particularly under partly cloudy conditions. More advanced anisotropic models such as the Perez model could improve the accuracy of front-side irradiance estimation.

(vi) Spectral Effects: The current model does not account for the spectral dependence of ground surface reflectivity or the wavelength-dependent response of bifacial silicon solar cells. As demonstrated by Riedel-Lyngskjaer et al. [6], spectral albedo effects can influence the actual electrical output from ground-reflected radiation, particularly for surfaces with strong spectral selectivity.

5.4  Scope for Future Work

Several promising directions for future research and development are identified based on the outcomes, insights, and limitations of this project:

(i) Extension to Annual Energy Yield: The current framework can be extended to perform year-round energy yield simulations by aggregating daily analyses across all days of the year. This extension would incorporate seasonal variations in solar geometry (declination, day length), ambient temperature profiles, and cloud cover patterns, enabling more robust design decisions that account for the full range of operating conditions experienced throughout the year. Annual yield predictions are essential for accurate financial analysis and investment decision-making in commercial PV projects.

(ii) Multi-Row Array Modeling: Incorporating inter-row shading models would enable the framework to optimize row-to-row spacing as an additional design variable alongside tilt angle, height, and albedo. This enhancement is particularly important for utility-scale bifacial solar farm design, where inter-row interactions significantly affect system performance and where ground coverage ratio optimization has substantial economic implications.

(iii) Machine Learning-Based Rapid Optimization: Developing machine learning models (such as neural networks, random forests, or gradient boosting machines) trained on comprehensive parametric sweep databases could enable near-instantaneous prediction of optimal configurations for new locations without requiring full computational analysis. Such surrogate models could dramatically reduce the optimization time while maintaining acceptable accuracy for preliminary system design.

(iv) Techno-Economic Analysis Integration: Integrating economic analysis capabilities would allow the framework to evaluate the cost-effectiveness of different bifacial PV configurations through levelized cost of energy (LCOE) calculations. This analysis would incorporate structural costs associated with different mounting heights, ground preparation costs for albedo enhancement measures, and the economic value of additional energy generation, enabling more holistic design optimization that considers both technical performance and financial viability.

(v) Real-Time Performance Monitoring: Developing a real-time monitoring and analytics system that continuously compares actual system performance with predicted values could enable adaptive optimization strategies and early detection of performance degradation. Integration with IoT sensors, edge computing devices, and cloud-based analytics platforms could provide automated performance assessment, fault detection, and predictive maintenance alerts.

(vi) Geographic Information System Integration: Combining the optimization framework with GIS tools and remote sensing data could enable automated site assessment for large-scale bifacial PV deployment planning. This integration would incorporate terrain elevation models, land use classification, local albedo mapping from satellite imagery, and proximity analysis to grid infrastructure, providing comprehensive site suitability analysis at regional and national scales.

(vii) Tracking System Analysis: Extending the framework to support single-axis and dual-axis tracking configurations would enable direct comparison of fixed-tilt and tracking bifacial PV systems under identical environmental conditions. Tracking systems can significantly enhance bifacial module performance by maintaining optimal front-side irradiance throughout the day while simultaneously affecting the shadow geometry and rear-side irradiance patterns.

(viii) Advanced Irradiance Models: Implementing anisotropic sky models (such as the Perez or Hay-Davies model) and spectral irradiance decomposition would improve the accuracy of both front-side and rear-side irradiance estimates, particularly for locations with complex sky conditions or for ground surfaces with strong spectral selectivity.

5.5  Task, Achievements, and Possible Beneficiaries

The primary task of this project was to develop and validate a computational framework for parametric analysis and optimization of bifacial PV installations using location-specific solar irradiance data. This task has been successfully accomplished through the design and implementation of an integrated platform comprising rigorous mathematical modeling, MATLAB/Simulink electrical simulation, and a web-based frontend interface with NASA POWER API integration.

The key achievements of this project include: (a) development and implementation of a validated mathematical model for bifacial PV irradiance computation incorporating view factor geometry and shadow effects; (b) creation of a MATLAB/Simulink model for electrical performance simulation of bifacial PV modules; (c) implementation of a user-friendly, web-based frontend platform with automated parametric sweep analysis capabilities; (d) demonstration of dual optimization (maximum energy and maximum rear gain) through a comprehensive case study; and (e) acceptance of a research paper based on this work at the IC2PCT 2026 International Conference.

The possible beneficiaries of this work include: PV system designers and engineers seeking optimal configuration guidance for bifacial installations; solar project developers evaluating the feasibility and expected performance enhancement of bifacial PV technology; EPC (Engineering, Procurement, and Construction) contractors requiring quick, reliable optimization guidance for specific project sites; academic researchers studying bifacial PV technology who need a flexible, validated simulation platform; and policymakers and energy planners assessing the potential of bifacial PV technology for meeting renewable energy targets.

REFERENCES

[1] B. Y. H. Liu and R. C. Jordan, "The interrelationship and characteristic distribution of direct, diffuse and total solar radiation," Solar Energy, vol. 4, no. 3, pp. 1-19, 1960.

[2] U. A. Yusufoglu, T. M. Pletzer, L. J. Koduvelikulathu, C. Comparotto, R. Kopecek, and H. Kurz, "Analysis of the annual performance of bifacial modules and optimization methods," IEEE J. Photovolt., vol. 5, no. 1, pp. 320-328, Jan. 2015.

[3] F. Dincer and E. Ozer, "Optimization of rear-side energy contribution in bifacial PV panels: A parametric analysis on albedo, tilt, height, and mounting configuration," Energies, vol. 18, no. 17, Art. no. 4443, 2025.

[4] K. Ganesan, D. P. Winston, S. Sugumar, and S. Jegan, "Performance analysis of n-type PERT bifacial solar PV module under diverse albedo conditions," Solar Energy, vol. 252, pp. 81-90, 2023.

[5] S. A. Pelaez, C. Deline, S. M. MacAlpine, B. Marion, J. S. Stein, and R. K. Kostuk, "Comparison of bifacial solar irradiance model predictions with field validation," IEEE J. Photovolt., vol. 9, no. 1, pp. 82-88, Jan. 2019.

[6] N. Riedel-Lyngskjaer, J. M. L. P. Larsen, and B. G. Nielsen, "Effect of spectral albedo on energy yield of bifacial photovoltaic systems," Solar Energy, vol. 231, pp. 176-186, 2022.

[7] R. Guerrero-Lemus, R. Vega, T. Kim, A. Kimm, and L. E. Shephard, "Bifacial solar photovoltaics - A technology review," Renew. Sustain. Energy Rev., vol. 60, pp. 1533-1549, 2016.

[8] M. Alam, M. R. Khan, and C. Deline, "Performance comparison between bifacial and monofacial photovoltaic systems under varying environmental conditions," Sustain. Energy Technol. Assess., vol. 57, Art. no. 103210, 2023.

[9] C. Deline, S. A. Pelaez, S. MacAlpine, and B. Marion, "Bifacial photovoltaic system performance: Separating fact from fiction," Nat. Renew. Energy Lab. (NREL), Golden, CO, USA, Tech. Rep. NREL/CP-5K00-74090, 2019.

[10] X. Sun, M. R. Khan, C. Deline, and M. A. Alam, "Optimization and performance of bifacial solar modules: A global perspective," Appl. Energy, vol. 212, pp. 1601-1610, 2018.

[11] A. Basak, S. Das, and R. Banerjee, "Tilt angle optimization for bifacial photovoltaic modules for enhanced energy generation," Appl. Energy, vol. 345, Art. no. 121245, 2025.

[12] U. Peter and M. Novak, "Optimal tilt angle for maximizing energy production of bifacial solar panels," J. Sustain. Energy Syst., vol. 14, no. 1, pp. 45-56, 2025.

[13] M. H. Aksoy and H. A. Ceylan, "Investigation of ground albedo effects on bifacial photovoltaic panel performance," Renew. Energy Res. J., vol. 13, no. 4, pp. 1800-1808, 2023.

[14] N. Baghel, A. Chandel, and S. K. Sharma, "Performance evaluation and optimization of albedo and tilt angle in bifacial photovoltaic systems," Solar Energy, vol. 250, pp. 84-95, 2023.

[15] I. N. Atalay, M. Kaya, and H. Ozturk, "Experimental investigation of ground surface reflectivity impact on bifacial photovoltaic systems," Solar Energy, vol. 246, pp. 65-74, 2023.

[16] D. S. Braga, R. Zilles, and E. Lorenzo, "Performance analysis of bifacial photovoltaic modules under tropical climatic conditions," Renew. Energy Environ. Sustain., vol. 8, pp. 1-10, 2023.

[17] A. F. Almarshoud, A. Al-Badi, and M. Al-Sulaiman, "Experimental performance analysis of bifacial photovoltaic modules under different operating conditions," Energies, vol. 17, no. 21, Art. no. 5456, 2024.

[18] M. T. Patel, M. R. Khan, and M. A. Alam, "Optimum design of tracking bifacial solar farms," IEEE J. Photovolt., vol. 11, no. 1, pp. 237-245, Jan. 2021.

[19] J. A. Duffie and W. A. Beckman, Solar Engineering of Thermal Processes, 4th ed. Hoboken, NJ, USA: Wiley, 2013.

[20] B. Marion, S. MacAlpine, C. Deline, A. Asgharzadeh, F. Toor, D. Riley, J. Stein, and C. Hansen, "A practical irradiance model for bifacial PV modules," in Proc. 44th IEEE Photovolt. Spec. Conf. (PVSC), Washington, DC, USA, 2017, pp. 1537-1542.

[21] P. K. Sahu, J. N. Roy, and C. Chakraborty, "Performance assessment of a bifacial PV system using a new energy estimation model," Solar Energy, vol. 262, Art. no. 111818, 2023.

[22] C. Ghenai, F. F. Ahmad, O. Rejeb, and A. K. Hamid, "Sensitivity analysis of design parameters and power gain correlations of bi-facial solar PV system using response surface methodology," Solar Energy, vol. 223, pp. 44-53, 2021.

APPENDIX A

MATLAB/Simulink Code

This appendix contains the MATLAB code used for the bifacial PV irradiance computation and parametric sweep analysis. The code implements the mathematical formulation described in Chapter 3 of the report.

A.1  Irradiance Calculation Function (calculate_irradiance.m)

function [G_effective, G_front, G_rear_effective] = calculate_irradiance( ...

GHI, Tilt, Height_cm, Albedo, Bifaciality, Latitude, Longitude, HourUTC, DayOfYear, PanelWidthM, PanelAzimuthDeg, RearStructureLossFraction)

% calculate_irradiance

% Physics-based bifacial irradiance model.

% Rear irradiance uses geometric shadow view factor F_V.

%

% Uses Liu-Jordan isotropic transposition for front,

% Erbs (1982) diffuse-fraction decomposition,

% and a 2D shadow projection + numerical integration for F_V.

%

% Inputs:

%   GHI            - Global Horizontal Irradiance (W/m^2 or Wh/m^2)

%   Tilt           - Panel tilt angle in degrees

%   Height_cm      - Panel ground clearance in cm

%   Albedo         - Ground reflectivity (0-1)

%   Bifaciality    - Rear-side contribution factor (0-1), default 0.7

%   Latitude       - Site latitude in degrees (positive N)

%   Longitude      - Site longitude in degrees (positive E)

%   HourUTC        - Hour of day in UTC (fractional, 0-24)

%   DayOfYear      - Day of year (1-366)

%   PanelWidthM    - Module width along the slope in meters, default 1.134

if nargin < 5  || isempty(Bifaciality),   Bifaciality   = 0.7;    end

if nargin < 6  || isempty(Latitude),      Latitude      = 28.47;  end

if nargin < 7  || isempty(Longitude),     Longitude     = 77.50;  end

if nargin < 8  || isempty(HourUTC),       HourUTC       = 12;     end

if nargin < 9  || isempty(DayOfYear),     DayOfYear     = 172;    end

if nargin < 10 || isempty(PanelWidthM),   PanelWidthM   = 1.134;  end

if nargin < 11 || isempty(PanelAzimuthDeg)

if Latitude >= 0

PanelAzimuthDeg = 180;

else

PanelAzimuthDeg = 0;

end

end

if nargin < 12 || isempty(RearStructureLossFraction), RearStructureLossFraction = 0.08; end

if GHI <= 0

G_effective = 0; G_front = 0; G_rear_effective = 0;

return;

end

beta_rad = deg2rad(Tilt);

lat_rad  = deg2rad(Latitude);

% --- Solar position ---

dec = 23.45 * sind(360/365 * (284 + DayOfYear));          % declination (deg)

B   = deg2rad(360/365 * (DayOfYear - 81));

eot = 9.87*sin(2*B) - 7.53*cos(B) - 1.5*sin(B);          % equation of time (min)

solar_time = HourUTC + Longitude/15 + eot/60;

ha  = (solar_time - 12) * 15;                              % hour angle (deg)

cos_zenith = sind(Latitude)*sind(dec) + cosd(Latitude)*cosd(dec)*cosd(ha);

if cos_zenith <= 0.01

G_effective = 0; G_front = 0; G_rear_effective = 0;

return;

end

% --- Solar elevation & azimuth ---

sun_elev_rad = asin(min(max(cos_zenith, 0), 1));

dec_rad = deg2rad(dec);

ha_rad  = deg2rad(ha);

sin_Z   = sqrt(max(0, 1 - cos_zenith^2));

if sin_Z < 1e-6

sun_az_rad = pi;

else

% Standard azimuth from NORTH:

%   cos(Az) = (sin(dec) - sin(elev)*sin(lat)) / (cos(elev)*cos(lat))

cos_az = (sin(dec_rad) - cos_zenith * sin(lat_rad)) / (sin_Z * cos(lat_rad));

sin_az = -cos(dec_rad) * sin(ha_rad) / sin_Z;

sun_az_rad = atan2(sin_az, cos_az);

if sun_az_rad < 0

sun_az_rad = sun_az_rad + 2*pi;

end

end

panel_az_rad = deg2rad(mod(PanelAzimuthDeg, 360));

% --- Erbs decomposition: GHI -> beam + diffuse ---

GSC   = 1361;

ecc   = 1 + 0.033 * cosd(360 * DayOfYear / 365);

G0h   = GSC * ecc * max(0, cos_zenith);

kt    = min(max(GHI / max(G0h, 1e-6), 0), 1.5);

if kt <= 0

kd = 1.0;

elseif kt <= 0.22

kd = 1.0 - 0.09 * kt;

elseif kt <= 0.80

kd = 0.9511 - 0.1604*kt + 4.388*kt^2 - 16.638*kt^3 + 12.336*kt^4;

else

kd = 0.165;

end

GHI_diff = GHI * kd;

GHI_beam = max(0, GHI - GHI_diff);

DNI      = GHI_beam / max(cos_zenith, 0.01);

% --- Front irradiance: generalized AOI + Liu-Jordan isotropic ---

sin_zenith = sqrt(max(0, 1 - cos_zenith^2));

cos_AOI = cos_zenith * cos(beta_rad) + sin_zenith * sin(beta_rad) * cos(sun_az_rad - panel_az_rad);

beam_tilted       = DNI * max(0, cos_AOI);

sky_vf            = (1 + cos(beta_rad)) / 2;

gnd_vf            = (1 - cos(beta_rad)) / 2;

diffuse_tilted    = GHI_diff * sky_vf;

ground_refl_front = GHI * Albedo * gnd_vf;

G_front           = max(0, beam_tilted + diffuse_tilted + ground_refl_front);

% --- Rear irradiance with geometric shadow attenuation ---

%   E_rear = alpha*DHI*(1+cos(beta))/2 + alpha*(GHI-DHI)*((1+cos(beta))/2 - F_V)

%   F_V = shadow view factor from 2D geometry + numerical integration

height_m = Height_cm / 100;

rear_vf  = (1 + cos(beta_rad)) / 2;

F_V = compute_shadow_view_factor(beta_rad, height_m, PanelWidthM, ...

sun_elev_rad, sun_az_rad, panel_az_rad);

rear_diffuse_comp = Albedo * GHI_diff * rear_vf;

rear_direct_comp  = Albedo * GHI_beam * max(0, rear_vf - F_V);

structure_factor = max(0, min(1, 1 - RearStructureLossFraction));

G_rear_effective  = max(0, (rear_diffuse_comp + rear_direct_comp) * Bifaciality * structure_factor);

G_effective = max(0, G_front + G_rear_effective);

end

function F_V = compute_shadow_view_factor(beta_rad, height_m, width_m, ...

sun_elev_rad, sun_az_rad, panel_az_rad)

% 2D shadow view factor using Simpson's rule numerical integration.

SIMPSON_N = 200;

% Solar profile angle

az_diff     = sun_az_rad - panel_az_rad;

cos_az_diff = cos(az_diff);

if sun_elev_rad <= 0.001 || cos_az_diff <= 0.001

F_V = 0;

return;

end

profile_angle = atan(tan(sun_elev_rad) / cos_az_diff);

if profile_angle <= 0.001

F_V = 0;

return;

end

tan_profile = tan(profile_angle);

if tan_profile <= 0.001

F_V = 0;

return;

end

% Shadow edges (x positive = behind panel, toward rear)

shadow_lower = height_m / tan_profile;

shadow_upper = width_m * cos(beta_rad) + (height_m + width_m * sin(beta_rad)) / tan_profile;

if shadow_upper <= shadow_lower + 1e-6

F_V = 0;

return;

end

% Module rear-surface centroid in 2D

%   Lower edge at (0, h), upper edge at (W*cos(beta), h + W*sin(beta))

mid_x = (width_m * cos(beta_rad)) / 2;

mid_y = height_m + (width_m * sin(beta_rad)) / 2;

% Rear-surface outward normal: (sin(beta), -cos(beta))

%   Panel tangent = (cos(beta), sin(beta))

%   Front normal (toward sun) = (-sin(beta), cos(beta))

%   Rear normal (away from sun) = (sin(beta), -cos(beta))

rear_nx = sin(beta_rad);

rear_ny = -cos(beta_rad);

% Simpson's rule

n  = SIMPSON_N;

dx = (shadow_upper - shadow_lower) / n;

total = vf_integrand(shadow_lower, mid_x, mid_y, rear_nx, rear_ny) + ...

vf_integrand(shadow_upper, mid_x, mid_y, rear_nx, rear_ny);

for i = 1:(n-1)

x_i = shadow_lower + i * dx;

if mod(i, 2) == 0

weight = 2;

else

weight = 4;

end

total = total + weight * vf_integrand(x_i, mid_x, mid_y, rear_nx, rear_ny);

end

f_shadow = (dx / 3) * total;

% F_V = view factor from rear to shadow strip (larger → more shadow blocking)

f_total = (1 + cos(beta_rad)) / 2;

F_V = max(0, min(f_shadow, f_total));

end

function val = vf_integrand(x, mid_x, mid_y, rear_nx, rear_ny)

vx = x - mid_x;

vy = -mid_y;

r  = sqrt(vx^2 + vy^2);

if r < 1e-9

val = 0;

return;

end

cos_theta1 = (rear_nx * vx + rear_ny * vy) / r;

cos_theta2 = mid_y / r;  % ground normal (0,1) dot (-vx,-vy)/r = mid_y/r

if cos_theta1 <= 0 || cos_theta2 <= 0

val = 0;

return;

end

val = (cos_theta1 * cos_theta2) / (2 * r);

end

A.2  Parametric Sweep Script (run_bifacial_sweep.m)

function run_bifacial_sweep(configPath, outputPath)

% run_bifacial_sweep

% MATLAB sweep runner invoked by backend.

%

% Usage:

%   run_bifacial_sweep('C:\path\config.json', 'C:\path\output.json')

if nargin < 2

error('run_bifacial_sweep requires configPath and outputPath.');

end

if ~isfile(configPath)

error('Config file not found: %s', configPath);

end

rawConfig = fileread(configPath);

cfg = jsondecode(rawConfig);

if ~isfield(cfg, 'irradiance') || ~isfield(cfg.irradiance, 'ghi')

error('Config must contain irradiance.ghi array.');

end

if ~isfield(cfg, 'irradiance') || ~isfield(cfg.irradiance, 'time')

error('Config must contain irradiance.time array.');

end

ghi = cfg.irradiance.ghi(:)';

time = cfg.irradiance.time;

if numel(ghi) ~= numel(time)

error('irradiance.ghi and irradiance.time must have equal length.');

end

tempC = [];

if isfield(cfg.irradiance, 'tempC')

tempC = cfg.irradiance.tempC(:)';

end

% Location (needed for solar position)

lat = 28.47; lon = 77.50;   % defaults

if isfield(cfg, 'location')

if isfield(cfg.location, 'latitude'),  lat = cfg.location.latitude;  end

if isfield(cfg.location, 'longitude'), lon = cfg.location.longitude; end

end

panel = cfg.panelConfig;

areaM2 = panel.areaM2;

frontEfficiency = panel.frontEfficiency;

inverterEfficiency = panel.inverterEfficiency;

bifaciality = panel.bifaciality;

if isfield(panel, 'widthM')

panelWidthM = panel.widthM;

else

panelWidthM = 1.134;

end

if isfield(panel, 'azimuthDeg')

panelAzimuthDeg = panel.azimuthDeg;

else

if lat >= 0

panelAzimuthDeg = 180;

else

panelAzimuthDeg = 0;

end

end

if isfield(panel, 'rearStructureLossFraction')

rearStructureLossFraction = panel.rearStructureLossFraction;

else

rearStructureLossFraction = 0.08;

end

if isfield(panel, 'noctC')

noctC = panel.noctC;

else

noctC = 45;

end

if isfield(panel, 'temperatureCoeffPerC')

temperatureCoeffPerC = panel.temperatureCoeffPerC;

else

temperatureCoeffPerC = -0.004;

end

heightValues = cfg.ranges.heightCm(:)';

tiltValues = cfg.ranges.tiltDeg(:)';

albedoValues = cfg.ranges.albedo(:)';

results = struct([]);

idx = 0;

for h = heightValues

for t = tiltValues

for a = albedoValues

idx = idx + 1;

[metrics, hourlySeries] = evaluate_config(ghi, time, h, t, a, ...

areaM2, frontEfficiency, inverterEfficiency, bifaciality, lat, lon, panelWidthM, panelAzimuthDeg, rearStructureLossFraction, tempC, noctC, temperatureCoeffPerC);

results(idx).configuration = struct('heightCm', h, 'tiltDeg', t, 'albedo', a); %#ok<AGROW>

results(idx).metrics = metrics; %#ok<AGROW>

results(idx).hourlySeries = hourlySeries; %#ok<AGROW>

end

end

end

if isempty(results)

error('No configuration combinations were generated.');

end

energy = arrayfun(@(x) x.metrics.totalEnergyKWh, results);

peak = arrayfun(@(x) x.metrics.peakPowerKW, results);

rear = arrayfun(@(x) x.metrics.rearGainPercent, results);

score = [-energy(:), -peak(:), -rear(:)];

[~, order] = sortrows(score, [1 2 3]);

sorted = results(order);

topN = min(10, numel(sorted));

top = sorted(1:topN);

best = sorted(1);

output = struct();

output.source = 'matlab-bifacial-sweep';

output.location = cfg.location;

output.dateRange = cfg.dateRange;

output.irradianceSource = cfg.irradianceSource;

output.panelModel = panel;

output.ranges = cfg.ranges;

output.combinationsTested = numel(sorted);

output.optimalConfiguration = map_summary(best, 1, true);

output.topConfigurations = arrayfun(@(i) map_summary(top(i), i, false), 1:topN, 'UniformOutput', false);

output.chartData = arrayfun(@(i) map_summary(sorted(i), i, false), 1:numel(sorted), 'UniformOutput', false);

output.matlabStatus = 'ok';

jsonText = jsonencode(output);

write_text_file(outputPath, jsonText);

end

function [metrics, hourlySeries] = evaluate_config(ghi, time, heightCm, tiltDeg, albedo, ...

areaM2, frontEfficiency, inverterEfficiency, bifaciality, lat, lon, panelWidthM, panelAzimuthDeg, rearStructureLossFraction, tempC, noctC, temperatureCoeffPerC)

totalEnergyKWh = 0;

peakPowerKW = 0;

totalFront = 0;

totalRear = 0;

totalGhi = 0;

totalEffective = 0;

hourlySeries = repmat(struct( ...

'time', '', ...

'ghiWm2', 0, ...

'frontEffectiveIrradianceWm2', 0, ...

'effectiveIrradianceWm2', 0, ...

'rearEffectiveIrradianceWm2', 0, ...

'powerKW', 0), 1, numel(ghi));

for i = 1:numel(ghi)

ghiVal = max(0, double(ghi(i)));

% Parse UTC hour and day-of-year from time string

tStr = read_time_value(time, i);

hourUTC = 12;  doy = 172;  % fallback defaults

try

dt = datetime(tStr, 'InputFormat', 'yyyy-MM-dd''T''HH:mm:ssXXX', 'TimeZone', 'UTC');

hourUTC = hour(dt) + minute(dt)/60;

doy = day(dt, 'dayofyear');

catch

try

dt = datetime(tStr, 'InputFormat', 'yyyy-MM-dd''T''HH:mm:ss', 'TimeZone', 'UTC');

hourUTC = hour(dt) + minute(dt)/60;

doy = day(dt, 'dayofyear');

catch

% use defaults

end

end

[effVal, frontVal, rearVal] = calculate_irradiance( ...

ghiVal, tiltDeg, heightCm, albedo, bifaciality, lat, lon, hourUTC, doy, panelWidthM, panelAzimuthDeg, rearStructureLossFraction);

if ~isempty(tempC) && numel(tempC) >= i && isfinite(tempC(i))

tAmb = tempC(i);

else

tAmb = 25;

end

cellTemp = tAmb + (effVal / 800) * (noctC - 20);

tempDerate = 1 + temperatureCoeffPerC * (cellTemp - 25);

tempDerate = max(0.5, min(1.15, tempDerate));

powerKW = (effVal / 1000) * areaM2 * frontEfficiency * inverterEfficiency * tempDerate;

totalEnergyKWh = totalEnergyKWh + powerKW;

peakPowerKW = max(peakPowerKW, powerKW);

totalFront = totalFront + frontVal;

totalRear = totalRear + rearVal;

totalGhi = totalGhi + ghiVal;

totalEffective = totalEffective + effVal;

hourlySeries(i).time = read_time_value(time, i);

hourlySeries(i).ghiWm2 = round3(ghiVal);

hourlySeries(i).frontEffectiveIrradianceWm2 = round3(frontVal);

hourlySeries(i).effectiveIrradianceWm2 = round3(effVal);

hourlySeries(i).rearEffectiveIrradianceWm2 = round3(rearVal);

hourlySeries(i).powerKW = round6(powerKW);

end

if totalGhi > 0

rearGainPercent = (totalRear / totalGhi) * 100;

else

rearGainPercent = 0;

end

if totalEffective > 0

frontSharePercent = (totalFront / totalEffective) * 100;

else

frontSharePercent = 0;

end

metrics = struct();

metrics.totalEnergyKWh = round6(totalEnergyKWh);

metrics.peakPowerKW = round6(peakPowerKW);

metrics.rearGainPercent = round4(rearGainPercent);

metrics.frontSharePercent = round4(frontSharePercent);

metrics.totalEffectiveIrradianceWhM2 = round3(totalEffective);

metrics.averageEffectiveIrradianceWm2 = round3(totalEffective / max(1, numel(ghi)));

end

function out = map_summary(entry, rankValue, includeHourly)

cfg = entry.configuration;

out = struct();

out.rank = rankValue;

out.configurationId = sprintf('H%s_T%s_A%s', num2str(cfg.heightCm), num2str(cfg.tiltDeg), num2str(cfg.albedo));

out.heightCm = cfg.heightCm;

out.tiltDeg = cfg.tiltDeg;

out.albedo = cfg.albedo;

out.totalEnergyKWh = entry.metrics.totalEnergyKWh;

out.peakPowerKW = entry.metrics.peakPowerKW;

out.rearGainPercent = entry.metrics.rearGainPercent;

out.frontSharePercent = entry.metrics.frontSharePercent;

out.averageEffectiveIrradianceWm2 = entry.metrics.averageEffectiveIrradianceWm2;

out.totalEffectiveIrradianceWhM2 = entry.metrics.totalEffectiveIrradianceWhM2;

if includeHourly

out.hourlySeries = entry.hourlySeries;

end

end

function write_text_file(pathValue, content)

fid = fopen(pathValue, 'w');

if fid == -1

error('Unable to open output file for writing: %s', pathValue);

end

cleanup = onCleanup(@() fclose(fid));

fwrite(fid, content, 'char');

end

function t = read_time_value(timeSeries, index)

if iscell(timeSeries)

raw = timeSeries{index};

if isstring(raw)

t = char(raw);

else

t = char(string(raw));

end

return;

end

if isstring(timeSeries)

t = char(timeSeries(index));

return;

end

if ischar(timeSeries)

t = timeSeries(index, :);

return;

end

t = char(string(timeSeries(index)));

end

function v = round3(x)

v = round(x, 3);

end

function v = round4(x)

v = round(x, 4);

end

function v = round6(x)

v = round(x, 6);

end

APPENDIX B

Backend Application Code

This appendix contains excerpts of the backend application code implemented in Node.js/Express for the NASA POWER API integration and simulation service.

B.1  Express Server Entry Point (server.js)

import cors from "cors";

import express from "express";

import path from "node:path";

import { fileURLToPath } from "node:url";

import "dotenv/config";

import simulationRoutes from "./routes/simulationRoutes.js";

const __filename = fileURLToPath(import.meta.url);

const __dirname = path.dirname(__filename);

const frontendPath = path.resolve(__dirname, "../../frontend");

const app = express();

const PORT = process.env.PORT || 4000;

app.use(express.json());

app.use(cors());

app.get("/", (req, res) => {

res.json({ success: true, message: "PV simulation backend is running" });

});

// Simple visual frontend at /app

app.use("/app", express.static(frontendPath));

app.use("/api/simulation", simulationRoutes);

app.listen(PORT, () => {

console.log(`Server listening on http://localhost:${PORT}`);

});

B.2  NASA POWER API Integration Service (irradianceService.js)

const POWER_BASE_URL = "https://power.larc.nasa.gov/api/temporal/hourly/point";

const POWER_PARAM_GHI = "ALLSKY_SFC_SW_DWN";

const POWER_PARAM_TEMP = "T2M";

const POWER_PARAMETERS = `${POWER_PARAM_GHI},${POWER_PARAM_TEMP}`;

const SUPPORTED_TIMEZONES = {

UTC: { offsetMinutes: 0, label: "UTC" },

IST: { offsetMinutes: 330, label: "IST (UTC+05:30)" }

};

const DEFAULT_TIMEZONE_KEY = "IST";

function createIrradianceError(message, statusCode = 500) {

const error = new Error(message);

error.statusCode = statusCode;

return error;

}

function formatDateCompact(date) {

const year = date.getUTCFullYear();

const month = String(date.getUTCMonth() + 1).padStart(2, "0");

const day = String(date.getUTCDate()).padStart(2, "0");

return `${year}${month}${day}`;

}

function formatDateIso(date) {

const year = date.getUTCFullYear();

const month = String(date.getUTCMonth() + 1).padStart(2, "0");

const day = String(date.getUTCDate()).padStart(2, "0");

return `${year}-${month}-${day}`;

}

function shiftUtcDateByDays(date, dayOffset) {

const shifted = new Date(date);

shifted.setUTCDate(shifted.getUTCDate() + dayOffset);

return shifted;

}

function parseInputDate(value, fieldName) {

if (!value) {

return null;

}

if (typeof value !== "string") {

throw createIrradianceError(`${fieldName} must be a string in YYYY-MM-DD format.`, 400);

}

const trimmed = value.trim();

const normalized = trimmed.replace(/-/g, "");

if (!/^\d{8}$/.test(normalized)) {

throw createIrradianceError(`${fieldName} must be in YYYY-MM-DD format.`, 400);

}

const year = Number(normalized.slice(0, 4));

const month = Number(normalized.slice(4, 6));

const day = Number(normalized.slice(6, 8));

const date = new Date(Date.UTC(year, month - 1, day));

const valid =

date.getUTCFullYear() === year &&

date.getUTCMonth() + 1 === month &&

date.getUTCDate() === day;

if (!valid) {

throw createIrradianceError(`${fieldName} is not a valid calendar date.`, 400);

}

return date;

}

function normalizeRequestedTimezone(rawTimezone) {

if (rawTimezone === undefined || rawTimezone === null || rawTimezone === "") {

return DEFAULT_TIMEZONE_KEY;

}

if (typeof rawTimezone !== "string") {

throw createIrradianceError("timezone must be a string (supported: UTC, IST).", 400);

}

const timezoneKey = rawTimezone.trim().toUpperCase();

if (!SUPPORTED_TIMEZONES[timezoneKey]) {

throw createIrradianceError("Unsupported timezone. Use UTC or IST.", 400);

}

return timezoneKey;

}

function getTimezoneOffsetMinutes(timezoneKey) {

return SUPPORTED_TIMEZONES[timezoneKey].offsetMinutes;

}

function getTodayInRequestedTimezone(timezoneKey) {

const offsetMinutes = getTimezoneOffsetMinutes(timezoneKey);

const shiftedNow = new Date(Date.now() + offsetMinutes * 60 * 1000);

return new Date(

Date.UTC(shiftedNow.getUTCFullYear(), shiftedNow.getUTCMonth(), shiftedNow.getUTCDate())

);

}

function normalizeDateRange(options = {}) {

const timezoneKey = normalizeRequestedTimezone(options.timezone);

const today = getTodayInRequestedTimezone("UTC");

const yesterday = shiftUtcDateByDays(today, -1);

const startInput = parseInputDate(options.startDate, "startDate");

const endInput = parseInputDate(options.endDate, "endDate");

const startDate = startInput || endInput || yesterday;

const endDate = endInput || startInput || yesterday;

if (startDate > endDate) {

throw createIrradianceError("startDate cannot be after endDate.", 400);

}

const millisecondsPerDay = 24 * 60 * 60 * 1000;

const dayCount = Math.floor((endDate - startDate) / millisecondsPerDay) + 1;

if (dayCount > 31) {

throw createIrradianceError("Date range too large. Keep it to 31 days or less.", 400);

}

return {

timezoneKey,

timezoneLabel: SUPPORTED_TIMEZONES[timezoneKey].label,

startDateIso: formatDateIso(startDate),

endDateIso: formatDateIso(endDate),

startDateCompact: formatDateCompact(startDate),

endDateCompact: formatDateCompact(endDate),

dayCount

};

}

function buildPowerUrl({ latitude, longitude, startDateCompact, endDateCompact, format }) {

const params = new URLSearchParams({

parameters: POWER_PARAMETERS,

community: "RE",

latitude: String(latitude),

longitude: String(longitude),

start: startDateCompact,

end: endDateCompact,

"time-standard": "UTC",

format

});

return `${POWER_BASE_URL}?${params.toString()}`;

}

function parseTimeStampToIso(hourKey) {

if (!/^\d{10}$/.test(hourKey)) {

return null;

}

const year = Number(hourKey.slice(0, 4));

const month = Number(hourKey.slice(4, 6));

const day = Number(hourKey.slice(6, 8));

const hour = Number(hourKey.slice(8, 10));

if (hour < 0 || hour > 23) {

return null;

}

const date = new Date(Date.UTC(year, month - 1, day, hour, 0, 0));

const valid =

date.getUTCFullYear() === year &&

date.getUTCMonth() + 1 === month &&

date.getUTCDate() === day &&

date.getUTCHours() === hour;

return valid ? date.toISOString() : null;

}

function formatOffsetTime(utcDate, offsetMinutes) {

const shifted = new Date(utcDate.getTime() + offsetMinutes * 60 * 1000);

const year = shifted.getUTCFullYear();

const month = String(shifted.getUTCMonth() + 1).padStart(2, "0");

const day = String(shifted.getUTCDate()).padStart(2, "0");

const hour = String(shifted.getUTCHours()).padStart(2, "0");

const minute = String(shifted.getUTCMinutes()).padStart(2, "0");

const second = String(shifted.getUTCSeconds()).padStart(2, "0");

const sign = offsetMinutes >= 0 ? "+" : "-";

const absOffsetMinutes = Math.abs(offsetMinutes);

const offsetHours = String(Math.floor(absOffsetMinutes / 60)).padStart(2, "0");

const offsetRemainderMinutes = String(absOffsetMinutes % 60).padStart(2, "0");

return `${year}-${month}-${day}T${hour}:${minute}:${second}${sign}${offsetHours}:${offsetRemainderMinutes}`;

}

function convertUtcIsoToTimezone(utcIso, timezoneKey) {

if (timezoneKey === "UTC") {

return {

timeIso: utcIso,

dateIso: utcIso.slice(0, 10)

};

}

const offsetMinutes = getTimezoneOffsetMinutes(timezoneKey);

const utcDate = new Date(utcIso);

if (Number.isNaN(utcDate.getTime())) {

return null;

}

const shiftedIso = formatOffsetTime(utcDate, offsetMinutes);

return {

timeIso: shiftedIso,

dateIso: shiftedIso.slice(0, 10)

};

}

function normalizePowerHourlyWhM2(rawValue, unitText) {

const value = Number(rawValue);

if (!Number.isFinite(value)) {

return null;

}

const normalizedUnit = String(unitText || "").toLowerCase();

if (normalizedUnit.includes("kw-hr") || normalizedUnit.includes("kwh")) {

return value * 1000;

}

return value;

}

export async function getIrradianceData(latitude, longitude, options = {}) {

const range = normalizeDateRange(options);

const jsonUrl = buildPowerUrl({

latitude,

longitude,

startDateCompact: range.startDateCompact,

endDateCompact: range.endDateCompact,

format: "JSON"

});

// csvUrl removed as unused

const response = await fetch(jsonUrl, {

headers: {

"User-Agent": process.env.IRRADIANCE_USER_AGENT || "pv-bifacial-sim/1.0"

}

});

if (!response.ok) {

throw createIrradianceError("Unable to fetch irradiance data", 502);

}

const payload = await response.json();

const ghiSeries = payload?.properties?.parameter?.[POWER_PARAM_GHI];

const tempSeries = payload?.properties?.parameter?.[POWER_PARAM_TEMP];

if (!ghiSeries || typeof ghiSeries !== "object") {

throw createIrradianceError("Irradiance data is incomplete for this location.", 502);

}

const fillValue = Number(payload?.properties?.fill_value ?? -999);

const rawUnit =

payload?.parameters?.[POWER_PARAM_GHI]?.units ||

payload?.header?.[POWER_PARAM_GHI]?.units ||

"kWh/m^2";

const sortedEntries = Object.entries(ghiSeries).sort(([a], [b]) => a.localeCompare(b));

const time = [];

const ghiWhM2Hourly = [];

const tempCHourly = [];

for (const [hourKey, rawValue] of sortedEntries) {

const numericRaw = Number(rawValue);

if (!Number.isFinite(numericRaw) || numericRaw === fillValue || numericRaw < 0) {

continue;

}

const isoTime = parseTimeStampToIso(hourKey);

if (!isoTime) {

continue;

}

const converted = normalizePowerHourlyWhM2(numericRaw, rawUnit);

if (converted === null) {

continue;

}

const zonedTime = convertUtcIsoToTimezone(isoTime, range.timezoneKey);

if (!zonedTime) {

continue;

}

time.push(zonedTime.timeIso);

ghiWhM2Hourly.push(converted);

// Extract temperature, default to 25C if missing/fill value

let tempVal = 25;

if (tempSeries && tempSeries[hourKey] !== undefined) {

const rawTemp = Number(tempSeries[hourKey]);

if (Number.isFinite(rawTemp) && rawTemp !== fillValue) {

tempVal = rawTemp;

}

}

tempCHourly.push(tempVal);

}

if (time.length === 0) {

throw createIrradianceError(

`No valid irradiance records were returned for ${range.startDateIso} to ${range.endDateIso}. Try an older UTC date range.`,

502

);

}

return {

hourly: {

time,

// Legacy key kept for compatibility with current simulation payload readers.

ghi_w_m2: ghiWhM2Hourly,

ghi_wh_m2: ghiWhM2Hourly,

temp_c: tempCHourly

},

units: {

ghiWhM2Hourly: "Wh/m^2",

totalGhiWhM2: "Wh/m^2",

averageEquivalentGhiWm2: "W/m^2",

peakHourlyGhiWhM2: "Wh/m^2",

// Legacy unit key kept for compatibility.

ghiWm2: "Wh/m^2",

tempC: "°C"

},

timezone: range.timezoneLabel,

dateRange: {

startDate: range.startDateIso,

endDate: range.endDateIso,

dayCount: range.dayCount

},

source: {

provider: "NASA POWER",

parameters: POWER_PARAMETERS,

parameter: POWER_PARAM_GHI,

rawUnit,

normalizedHourlyUnit: "Wh/m^2",

queriedTimeStandard: "UTC",

jsonUrl,

csvUrl: jsonUrl // Placeholder or remove key

}

};

}

B.3  Simulation Engine - Solar Geometry and Shadow View Factor (simulationService.js, excerpt)

const DEFAULT_RANGE_SPECS = {

heightCm: { min: 50, max: 450, step: 50 },

tiltDeg: { min: 10, max: 50, step: 10 },

albedo: { min: 0.2, max: 0.6, step: 0.1 }

};

const RANGE_LIMITS = {

heightCm: { min: 10, max: 500, decimals: 0, maxValues: 50 },

tiltDeg: { min: 0, max: 85, decimals: 1, maxValues: 60 },

albedo: { min: 0.05, max: 0.95, decimals: 2, maxValues: 40 }

};

const DEFAULT_PANEL_CONFIG = {

areaM2: 2.2,

widthM: 1.134,

frontEfficiency: 0.21,

inverterEfficiency: 0.96,

bifaciality: 0.7,

azimuthDeg: 180,

rearStructureLossFraction: 0.08,

noctC: 45,

temperatureCoeffPerC: -0.004

};

const MAX_CONFIGURATIONS = 10000;

const TOP_CONFIGURATION_LIMIT = 10;

const IV_PV_CURVE_LIMIT = 5;

const IV_PV_POINTS = 80;

const VOC_REFERENCE_V = 64.5;

const VMPP_RATIO_TARGET = 0.82;

const IMPP_RATIO_TARGET = 0.92;

const KNEE_EXPONENT_MIN = 8;

const KNEE_EXPONENT_MAX = 16;

function createInputError(message) {

const error = new Error(message);

error.statusCode = 400;

return error;

}

function roundTo(value, decimals) {

return Number(value.toFixed(decimals));

}

function clamp(value, min, max) {

return Math.min(max, Math.max(min, value));

}

function toFiniteNumber(value, fallback = 0) {

const parsed = Number(value);

return Number.isFinite(parsed) ? parsed : fallback;

}

// ─── Solar geometry helpers ─────────────────

function degToRad(deg) {

return (deg * Math.PI) / 180;

}

function getDayOfYear(isoString) {

const d = new Date(isoString);

const start = new Date(Date.UTC(d.getUTCFullYear(), 0, 1));

return Math.floor((d - start) / 86400000) + 1;

}

function solarDeclination(doy) {

// declination approximation

return 23.45 * Math.sin(degToRad((360 / 365) * (284 + doy)));

}

function equationOfTime(doy) {

// equation-of-time approximation

const B = degToRad((360 / 365) * (doy - 81));

return 9.87 * Math.sin(2 * B) - 7.53 * Math.cos(B) - 1.5 * Math.sin(B);

}

function cosSolarZenith(latDeg, decDeg, hourAngleDeg) {

const latR = degToRad(latDeg);

const decR = degToRad(decDeg);

const haR = degToRad(hourAngleDeg);

return Math.sin(latR) * Math.sin(decR) + Math.cos(latR) * Math.cos(decR) * Math.cos(haR);

}

function cosAngleOfIncidence(cosZ, tiltDeg, sunAzimuthRad, panelAzimuthRad) {

// Generalized AOI for any surface azimuth.

const betaRad = degToRad(tiltDeg);

const sinZ = Math.sqrt(Math.max(0, 1 - cosZ * cosZ));

return (

cosZ * Math.cos(betaRad) +

sinZ * Math.sin(betaRad) * Math.cos(sunAzimuthRad - panelAzimuthRad)

);

}

function extraterrestrialHorizontal(doy, cosZenith) {

const GSC = 1361; // W/m² solar constant

const eccCorr = 1 + 0.033 * Math.cos(degToRad((360 * doy) / 365));

return GSC * eccCorr * Math.max(0, cosZenith);

}

// ─── Diffuse fraction calculation (kt-based)

function diffuseFraction(kt) {

// piecewise kd formula

if (kt <= 0) return 1.0;

if (kt <= 0.22) return 1.0 - 0.09 * kt;

if (kt <= 0.80) {

return (

0.9511 -

0.1604 * kt +

4.388 * kt ** 2 -

16.638 * kt ** 3 +

12.336 * kt ** 4

);

}

return 0.165;

}

// ─── Solar azimuth (used for shadow projection) ─────────────────

function solarAzimuth(latRad, decRad, haRad, cosZ) {

// Returns solar azimuth in radians, measured clockwise from north.

// South = π (≈180°), East ≈ 90°–135°, West ≈ 225°–270°.

const sinZ = Math.sqrt(Math.max(0, 1 - cosZ * cosZ));

if (sinZ < 1e-6) return Math.PI; // sun at zenith → default south

// Standard solar azimuth from NORTH:

//   cos(Az) = (sin(δ) − sin(h)·sin(φ)) / (cos(h)·cos(φ))

// where h = elevation (sin(h) = cosZ), cos(h) = sinZ

const cosAz = (Math.sin(decRad) - cosZ * Math.sin(latRad)) / (sinZ * Math.cos(latRad));

const sinAz = -Math.cos(decRad) * Math.sin(haRad) / sinZ;

let az = Math.atan2(sinAz, cosAz); // range -π..π, 0 = north

if (az < 0) az += 2 * Math.PI;     // normalize to 0..2π

return az;

}

// ─── Shadow View Factor F_V ──────────────

//

// Computes the view factor from the module's rear surface to the shadow

// that the module itself casts on the ground.  The shadow attenuates

// the *direct* component of ground-reflected radiation only.

//

// Uses 2D cross-section geometry + Simpson's rule numerical integration.

const SIMPSON_POINTS = 200; // must be even

function calculateShadowViewFactor(betaRad, heightM, widthM, sunElevationRad, sunAzimuthRad, panelAzimuthRad) {

// ── Solar profile angle: sun elevation projected onto the panel's

//    cross-sectional (meridian) plane.

const azDiff = sunAzimuthRad - panelAzimuthRad;

const cosAzDiff = Math.cos(azDiff);

if (sunElevationRad <= 0.001 || cosAzDiff <= 0.001) return 0;

const profileAngle = Math.atan(Math.tan(sunElevationRad) / cosAzDiff);

if (profileAngle <= 0.001) return 0;

const tanProfile = Math.tan(profileAngle);

if (tanProfile <= 0.001) return 0;

// ── 2D coordinate system (cross-section perpendicular to panel length):

//    x-axis = horizontal, positive toward the REAR side (north for south-facing)

//    y-axis = vertical, positive upward

//

//    Panel lower edge at (0, h)

//    Panel upper edge at (W·cosβ, h + W·sinβ)  ← goes toward rear and up

//    Shadow falls in the +x direction (behind the panel)

// ── Shadow edges on the ground (horizontal distance in +x from panel base)

const shadowLower = heightM / tanProfile;

const shadowUpper = widthM * Math.cos(betaRad) + (heightM + widthM * Math.sin(betaRad)) / tanProfile;

if (shadowUpper <= shadowLower + 1e-6) return 0;

// ── Module rear-surface centroid

const midX = (widthM * Math.cos(betaRad)) / 2;

const midY = heightM + (widthM * Math.sin(betaRad)) / 2;

// Rear-surface outward normal direction:

// Panel tangent from lower to upper = (cosβ, sinβ)

// Front normal (faces sun/south) = (-sinβ, cosβ)

// Rear normal (faces ground/north) = (sinβ, -cosβ)

const rearNormalX = Math.sin(betaRad);

const rearNormalY = -Math.cos(betaRad);

// ── Simpson's rule integration of F_shadow over [shadowLower, shadowUpper]

const n = SIMPSON_POINTS;

const dx = (shadowUpper - shadowLower) / n;

function integrand(x) {

// Vector from module centroid to ground point (x, 0)

const vx = x - midX;

const vy = -midY;

const r = Math.sqrt(vx * vx + vy * vy);

if (r < 1e-9) return 0;

// cos θ₁: angle between rear normal and line to ground point

const cosTheta1 = (rearNormalX * vx + rearNormalY * vy) / r;

// cos θ₂: angle between ground normal (0,1) and line from ground to module

//          = (0·(-vx) + 1·(-vy)) / r = midY / r

const cosTheta2 = midY / r;

if (cosTheta1 <= 0 || cosTheta2 <= 0) return 0;

return (cosTheta1 * cosTheta2) / (2 * r);

}

let sum = integrand(shadowLower) + integrand(shadowUpper);

for (let i = 1; i < n; i++) {

const x = shadowLower + i * dx;

sum += integrand(x) * (i % 2 === 0 ? 2 : 4);

}

const fShadow = (dx / 3) * sum;

// ── F_V = view factor from rear surface to the shadow strip.

//    When F_V is large → shadow blocks more direct reflection → less rear irradiance.

//    Clamp to valid range [0, (1+cosβ)/2].

const fTotal = (1 + Math.cos(betaRad)) / 2;

return clamp(fShadow, 0, fTotal);

}

B.4  Simulation Engine - Effective Irradiance Calculation (simulationService.js, excerpt)

// Computes front and rear effective irradiance using transposition,

// diffuse decomposition, and geometric rear-side shadow/view-factor terms.

function calculateEffectiveIrradiance({

ghiWm2, tiltDeg, heightCm, albedo, bifaciality,

latitude, longitude, timeIso, panelWidthM, panelAzimuthDeg,

rearStructureLossFraction, ambientTempC, noctC, temperatureCoeffPerC

}) {

if (ghiWm2 <= 0) {

return { frontEffectiveWm2: 0, rearEffectiveWm2: 0, totalEffectiveWm2: 0, tempDerate: 1 };

}

const betaRad = degToRad(tiltDeg);

const latRad = degToRad(latitude);

// ── Solar position ──

const d = new Date(timeIso);

const hourUTC = d.getUTCHours() + d.getUTCMinutes() / 60;

const doy = getDayOfYear(timeIso);

const dec = solarDeclination(doy);

const eot = equationOfTime(doy);

const solarTime = hourUTC + longitude / 15 + eot / 60;

const ha = (solarTime - 12) * 15; // hour angle in degrees

const cosZ = cosSolarZenith(latitude, dec, ha);

if (cosZ <= 0.01) {

// Sun below horizon

return { frontEffectiveWm2: 0, rearEffectiveWm2: 0, totalEffectiveWm2: 0, tempDerate: 1 };

}

// ── Solar elevation & azimuth ──

const sunElevationRad = Math.asin(clamp(cosZ, 0, 1));

const decRad = degToRad(dec);

const haRad = degToRad(ha);

const sunAzimuthRad = solarAzimuth(latRad, decRad, haRad, cosZ);

const panelAzimuthRad = degToRad(((panelAzimuthDeg % 360) + 360) % 360);

// ── Erbs decomposition: GHI → beam + diffuse ──

const G0h = extraterrestrialHorizontal(doy, cosZ);

const kt = G0h > 0 ? clamp(ghiWm2 / G0h, 0, 1.5) : 0;

const kd = diffuseFraction(kt);

const ghiDiffuse = ghiWm2 * kd;

const ghiBeam = Math.max(0, ghiWm2 - ghiDiffuse);

const dni = cosZ > 0.01 ? ghiBeam / cosZ : 0;

// ── Front irradiance: tilted-plane + Liu-Jordan isotropic transposition ──

const cosI = cosAngleOfIncidence(cosZ, tiltDeg, sunAzimuthRad, panelAzimuthRad);

const beamTilted = dni * Math.max(0, cosI);

const skyVF = (1 + Math.cos(betaRad)) / 2;

const gndVF = (1 - Math.cos(betaRad)) / 2;

const diffuseTilted = ghiDiffuse * skyVF;

const groundReflFront = ghiWm2 * albedo * gndVF;

const frontBase = Math.max(0, beamTilted + diffuseTilted + groundReflFront);

const frontEffectiveWm2 = frontBase;

// ── Rear irradiance with geometric shadow attenuation ──

//

//   E_rear = α·DHI·(1+cosβ)/2  +  α·(GHI-DHI)·((1+cosβ)/2 − F_V)

//

//   F_V = shadow view factor, computed from 2D geometry + numerical integration.

//   The result is then multiplied by bifaciality.

const heightM = heightCm / 100;

const rearVF = (1 + Math.cos(betaRad)) / 2; // rear-side ground view factor

const Fv = calculateShadowViewFactor(

betaRad, heightM, panelWidthM || DEFAULT_PANEL_CONFIG.widthM,

sunElevationRad, sunAzimuthRad, panelAzimuthRad

);

const rearDiffuseComponent = albedo * ghiDiffuse * rearVF;

const rearDirectComponent = albedo * ghiBeam * Math.max(0, rearVF - Fv);

// Apply realistic structure-related rear loss (rails, clamps, junction box, etc.).

const structureFactor = clamp(1 - toFiniteNumber(rearStructureLossFraction, 0), 0, 1);

const rearEffectiveWm2 = Math.max(

0,

(rearDiffuseComponent + rearDirectComponent) * bifaciality * structureFactor

);

const totalIrr = frontEffectiveWm2 + rearEffectiveWm2;

// ── Temperature Derating (NOCT Model) ──

const NOCT = toFiniteNumber(noctC, DEFAULT_PANEL_CONFIG.noctC);

const tAmb = ambientTempC !== undefined ? ambientTempC : 25;

const cellTemp = tAmb + (totalIrr / 800) * (NOCT - 20);

const gamma = toFiniteNumber(

temperatureCoeffPerC,

DEFAULT_PANEL_CONFIG.temperatureCoeffPerC

);

// Derating factor relative to STC (25°C)

let tempDerate = 1 + gamma * (cellTemp - 25);

tempDerate = Math.min(Math.max(tempDerate, 0.5), 1.15);

return { frontEffectiveWm2, rearEffectiveWm2, totalEffectiveWm2: Math.max(0, totalIrr), tempDerate };

}

function buildConfigurationLabel(configuration) {

return `H${configuration.heightCm}_T${configuration.tiltDeg}_A${configuration.albedo}`;

}

function evaluateConfiguration({ configuration, irradiance, panel, location }) {

let totalEnergyKWh = 0;

let peakPowerKW = 0;

let totalFrontEffectiveWm2 = 0;

let totalRearEffectiveWm2 = 0;

let totalGhiWm2 = 0;

let totalEffectiveWm2 = 0;

const hourlySeries = [];

const tempArray = irradiance.tempC || new Array(irradiance.time.length).fill(25);

for (let index = 0; index < irradiance.time.length; index += 1) {

const ghiWm2 = Math.max(0, toFiniteNumber(irradiance.ghi[index], 0));

const ambientTempC = tempArray[index];

const effective = calculateEffectiveIrradiance({

ghiWm2,

tiltDeg: configuration.tiltDeg,

heightCm: configuration.heightCm,

albedo: configuration.albedo,

bifaciality: panel.bifaciality,

latitude: location.latitude,

longitude: location.longitude,

timeIso: irradiance.time[index],

panelWidthM: panel.widthM,

panelAzimuthDeg: panel.azimuthDeg,

rearStructureLossFraction: panel.rearStructureLossFraction,

ambientTempC,

noctC: panel.noctC,

temperatureCoeffPerC: panel.temperatureCoeffPerC

});

const powerKW =

(effective.totalEffectiveWm2 / 1000) *

panel.areaM2 *

panel.frontEfficiency *

panel.inverterEfficiency *

effective.tempDerate;

totalEnergyKWh += powerKW;

peakPowerKW = Math.max(peakPowerKW, powerKW);

totalGhiWm2 += ghiWm2;

totalFrontEffectiveWm2 += effective.frontEffectiveWm2;

totalRearEffectiveWm2 += effective.rearEffectiveWm2;

totalEffectiveWm2 += effective.totalEffectiveWm2;

hourlySeries.push({

time: irradiance.time[index],

ghiWm2: roundTo(ghiWm2, 3),

frontEffectiveIrradianceWm2: roundTo(effective.frontEffectiveWm2, 3),

effectiveIrradianceWm2: roundTo(effective.totalEffectiveWm2, 3),

rearEffectiveIrradianceWm2: roundTo(effective.rearEffectiveWm2, 3),

powerKW: roundTo(powerKW, 6)

});

}

// Rear and front shares should both be fractions of the total effective

// irradiance so that rear + front ≈ 100% (when totalEffectiveWm2 > 0).

const rearGainPercent =

totalEffectiveWm2 > 0 ? (totalRearEffectiveWm2 / totalEffectiveWm2) * 100 : 0;

const frontSharePercent =

totalEffectiveWm2 > 0 ? (totalFrontEffectiveWm2 / totalEffectiveWm2) * 100 : 0;

return {

configuration,

metrics: {

totalEnergyKWh: roundTo(totalEnergyKWh, 6),

peakPowerKW: roundTo(peakPowerKW, 6),

rearGainPercent: roundTo(rearGainPercent, 4),

frontSharePercent: roundTo(frontSharePercent, 4),

totalEffectiveIrradianceWhM2: roundTo(totalEffectiveWm2, 3),

averageEffectiveIrradianceWm2: roundTo(totalEffectiveWm2 / irradiance.time.length, 3)

},

hourlySeries

};

}

function compareConfigurations(left, right) {

LIST OF PUBLICATIONS

The following publication has been produced as part of this project work:

[1] A. Upadhyay, P. Chakrabarty, Aman, and A. Garg, "Performance analysis of bifacial PV system based on different albedos," accepted for presentation at IC2PCT 2026 (International Conference on Clean and Prospective Computing Technologies), 2026.

BIO-DATA

Ashish Upadhyay

Roll Number: 2022UEE4521

Programme: B.Tech. Electrical Engineering

Department: Electrical Engineering

Institution: Netaji Subhas University of Technology (NSUT), Dwarka, New Delhi - 110078

Batch: 2022-2026

Pratyai Chakrabarty

Roll Number: 2022UEE4586

Programme: B.Tech. Electrical Engineering

Department: Electrical Engineering

Institution: Netaji Subhas University of Technology (NSUT), Dwarka, New Delhi - 110078

Batch: 2022-2026

Aman

Roll Number: 2022UEE4532

Programme: B.Tech. Electrical Engineering

Department: Electrical Engineering

Institution: Netaji Subhas University of Technology (NSUT), Dwarka, New Delhi - 110078

Batch: 2022-2026

Aditya Garg

Roll Number: 2022UEE4503

Programme: B.Tech. Electrical Engineering

Department: Electrical Engineering

Institution: Netaji Subhas University of Technology (NSUT), Dwarka, New Delhi - 110078

Batch: 2022-2026